AXI4 PROTOCOL:

AXI-MASTER:

module axi_master(

input clk,

input rst,

input start_write,

input start_read,

output reg [31:0] AWADDR,

output reg [7:0]  AWLEN,

output reg [1:0]  AWBURST,

output reg        AWVALID,

input             AWREADY,

output reg [31:0] WDATA,

output reg        WLAST,

output reg        WVALID,

input             WREADY,

input  [1:0] BRESP,

input        BVALID,

output reg   BREADY,

output reg [31:0] ARADDR,

output reg [7:0]  ARLEN,

output reg [1:0]  ARBURST,

output reg        ARVALID,

input             ARREADY,

input  [31:0] RDATA,

input         RLAST,

input         RVALID,

output reg    RREADY

);

reg [7:0] count;

reg [7:0] burst_len;

always @(posedge clk) begin

if (rst) begin

AWVALID<=0; WVALID<=0; BREADY<=1;

ARVALID<=0; RREADY<=0;

WLAST<=0; count<=0;

WDATA<=0;

end

else begin

// ---------------- WRITE ADDRESS ----------------

if (start_write && !AWVALID && !WVALID) begin

AWADDR  <= 32'h20;

AWLEN   <= 3;

AWBURST <= 2'b01;

AWVALID <= 1;

$display("[%0t] MASTER -> SLAVE : WRITE ADDR = %08h LEN=%0d BURST=%02b",

$time, 32'h20, 3, 2'b01);

end

if (AWVALID && AWREADY) begin

AWVALID   <= 0;

WVALID    <= 1;

burst_len <= AWLEN;

count     <= 0;

// LOAD FIRST DATA BEFORE HANDSHAKE

WDATA <= 32'hA0000000;

WLAST <= (AWLEN == 0);

end

// ---------------- WRITE DATA ----------------

if (WVALID && WREADY) begin

$display("[%0t] MASTER -> SLAVE : WRITE DATA = %08h WLAST=%0d",

$time, WDATA, WLAST);

if (count == burst_len) begin

WVALID <= 0;

end

else begin

count <= count + 1;

WDATA <= 32'hA0000000 + (count + 1);

WLAST <= ((count + 1) == burst_len);

end

end

if (BVALID)

$display("[%0t] SLAVE  -> MASTER: WRITE RESP = %02b",

$time, BRESP);

// ---------------- READ ADDRESS ----------------

if (start_read && !ARVALID) begin

ARADDR  <= 32'h20;

ARLEN   <= 3;

ARBURST <= 2'b01;

ARVALID <= 1;

$display("[%0t] MASTER -> SLAVE : READ ADDR = %08h LEN=%0d BURST=%02b",

$time, 32'h20, 3, 2'b01);

end

if (ARVALID && ARREADY) begin

ARVALID <= 0;

RREADY  <= 1;

end

if (RVALID && RREADY) begin

$display("[%0t] SLAVE  -> MASTER: READ DATA = %08h RLAST=%0d",

$time, RDATA, RLAST);

if (RLAST)

RREADY <= 0;

end

end

end

endmodule

AXI-SLAVE:

module axi_slave #(

parameter ADDR_WIDTH = 32,

parameter DATA_WIDTH = 32,

parameter MEM_DEPTH  = 256

)(

input                    clk,

input                    rst,

// ---------------- WRITE ADDRESS ----------------

input                    AWVALID,

input  [ADDR_WIDTH-1:0]  AWADDR,

input  [7:0]             AWLEN,

output reg               AWREADY,

// ---------------- WRITE DATA ----------------

input                    WVALID,

input  [DATA_WIDTH-1:0]  WDATA,

input                    WLAST,

output reg               WREADY,

// ---------------- WRITE RESPONSE ----------------

output reg               BVALID,

output reg [1:0]         BRESP,

input                    BREADY,

// ---------------- READ ADDRESS ----------------

input                    ARVALID,

input  [ADDR_WIDTH-1:0]  ARADDR,

input  [7:0]             ARLEN,

output reg               ARREADY,

// ---------------- READ DATA ----------------

output reg               RVALID,

output reg [DATA_WIDTH-1:0] RDATA,

output reg               RLAST,

input                    RREADY

);

// ---------------- MEMORY ----------------

reg [DATA_WIDTH-1:0] mem [0:MEM_DEPTH-1];

integer i;

initial begin

for (i=0; i<MEM_DEPTH; i=i+1)

mem[i] = 0;

end

// ---------------- INTERNAL REGISTERS ----------------

reg [ADDR_WIDTH-1:0] addr_reg;

reg [7:0] burst_len;

reg [7:0] write_count;

reg [7:0] read_count;

reg write_active;

reg read_active;

// =====================================================

//                    WRITE CHANNEL

// =====================================================

always @(posedge clk) begin

if (rst) begin

AWREADY <= 1;

WREADY  <= 1;

BVALID  <= 0;

BRESP   <= 0;

write_active <= 0;

write_count  <= 0;

end

else begin

// Address handshake

if (AWVALID && AWREADY) begin

addr_reg     <= AWADDR;

burst_len    <= AWLEN;

write_count  <= 0;

write_active <= 1;

end

// Data handshake

if (write_active && WVALID && WREADY) begin

mem[(addr_reg >> 2) + write_count] <= WDATA;

write_count <= write_count + 1;

if (WLAST) begin

write_active <= 0;

BVALID <= 1;

BRESP  <= 2'b00;  // OKAY response

end

end

// Response handshake

if (BVALID && BREADY)

BVALID <= 0;

end

end

// =====================================================

//                    READ CHANNEL (FIXED)

// =====================================================

always @(posedge clk) begin

if (rst) begin

ARREADY     <= 1;

RVALID      <= 0;

RLAST       <= 0;

read_active <= 0;

read_count  <= 0;

RDATA       <= 0;

end

else begin

// Address handshake

if (ARVALID && ARREADY) begin

addr_reg    <= ARADDR;

burst_len   <= ARLEN;

read_count  <= 0;

read_active <= 1;

RVALID <= 1;

RDATA  <= mem[(ARADDR >> 2)];

RLAST  <= (ARLEN == 0);

end

// Data phase

else if (read_active && RVALID && RREADY) begin

if (read_count == burst_len) begin

RVALID      <= 0;

read_active <= 0;

RLAST       <= 0;

end

else begin

read_count <= read_count + 1;

RDATA <= mem[(addr_reg >> 2) + read_count + 1];

RLAST <= ((read_count + 1) == burst_len);

end

end

end

end

endmodule

AXI-TB:

module tb_axi_top;

reg clk=0;

reg rst=1;

reg start_write=0;

reg start_read=0;

always #5 clk = ~clk;

// AXI wires

wire [31:0] AWADDR;

wire [7:0]  AWLEN;

wire [1:0]  AWBURST;

wire        AWVALID;

wire        AWREADY;

wire [31:0] WDATA;

wire        WLAST;

wire        WVALID;

wire        WREADY;

wire [1:0]  BRESP;

wire        BVALID;

wire        BREADY;

wire [31:0] ARADDR;

wire [7:0]  ARLEN;

wire [1:0]  ARBURST;

wire        ARVALID;

wire        ARREADY;

wire [31:0] RDATA;

wire        RLAST;

wire        RVALID;

wire        RREADY;

// Instantiate

axi_master master(

clk,rst,start_write,start_read,

AWADDR,AWLEN,AWBURST,AWVALID,AWREADY,

WDATA,WLAST,WVALID,WREADY,

BRESP,BVALID,BREADY,

ARADDR,ARLEN,ARBURST,ARVALID,ARREADY,

RDATA,RLAST,RVALID,RREADY

);

axi_slave slave(

.clk(clk),

.rst(rst),

.AWVALID(AWVALID),

.AWADDR(AWADDR),

.AWLEN(AWLEN),

.AWREADY(AWREADY),

.WVALID(WVALID),

.WDATA(WDATA),

.WLAST(WLAST),

.WREADY(WREADY),

.BVALID(BVALID),

.BRESP(BRESP),

.BREADY(BREADY),

.ARVALID(ARVALID),

.ARADDR(ARADDR),

.ARLEN(ARLEN),

.ARREADY(ARREADY),

.RDATA(RDATA),

.RVALID(RVALID),

.RLAST(RLAST),

.RREADY(RREADY)

);

initial begin

#20 rst=0;

#20 start_write=1;

#10 start_write=0;

#80 start_read=1;

#10 start_read=0;

#1000 $finish;

end

endmodule